MSX Tape I/O

-------------

BIT AND BYTE ENCODING

Information on tape is stored as square waves of 1200 or 2400 baud. The

encoding used is FSK (frequency shifting key). The next table shows the

relation between the selected recording speed, the bits and the pulses

put on tape:

bps   bit   encoding          freq (Hz)

_______

1200   0    _______|       |  1200

___     ___

1200   1    ___|   |___|   |  2400

___

2400   0    ___|   |          2400

_   _

2400   1    _| |_| |          4800

Each byte is put on tape as a sequence of 11 bits:

0 b0 b1 b2 b3 b4 b5 b6 b7 1 1  (one startbit, 2 stopbits, LSb first)

Futhermore 2 kinds of headers are used: short and long, a long header being

4 times as long as a short one. They consist of a series of square wave pulses

(bit value 1):

bps   header   freq (Hz)   nr of pulses

1200  short    2400        4000             (5/3  sec)

1200  long     2400        16000            (20/3 sec)

2400  short    4800        8000

2400  long     4800        32000

FILE FORMATS

Files on tape are stored in two blocks. The first block is the filedescriptor,

the second contains the filedata.  Three types of files exist:

BASIC file, memory dump and data file.

BASIC File (CSAVE/CLOAD)

filedescriptor: long header

10x #D3       (filetype)

6 bytes filename

file data:      short header

tokenized BASIC

7x #00

memory dump (BSAVE/BLOAD)

filedescriptor: long header

10x #D0

6 bytes filename

file data:      short header

2 bytes start address

2 bytes end address

2 bytes exec address

memory dump

data file (SAVE/LOAD/OPEN)

filedescriptor: long header

10x #EA

6 bytes filename

file data:      short header

block of 256 bytes

short header

block of 256 bytes

....

last block contains 256x #1A (^Z/EoF)